How to use the time-lapse photography function manual of the camera

Function Introduction: The time-lapse photography function refers to the recording of time in miniature. For example, recording a 30 minute video on a camera only takes up 1 minute of recording time or storage space. When playing a 30 minute video file, the video file is fast forwarded and played within 1 minute.

How to set the time-lapse camera function:

- Tools required:

1* computer,

1* camera with time-lapse function

1* micro SD card (TF card) with a maximum capacity of 512GB

Camera search tool software: Search Tools_V5.8_20240929.exe

- Insert the SD card into the SD card slot of the camera before connecting to the power supply, and connect the camera to the network through a network cable. The computer also needs to be connected to the same network as the camera.

- Install the camera search tool software:

- After successful installation, run the camera search tool software as shown in the following figure

- Click the ”Start search” button to search for the camera. If the IP address of the camera is incorrect, select the camera you want to set up, check the box in front of the IP address, and then click the camera path setting button. As shown in the figure below, the camera IP address will be automatically changed to be on the same LAN as your computer's IP address.

- Select the camera, then right-click and select "Remote config>English" from the pop-up menu to enter the remote configuration menu. As shown in the following figure:

- After entering the remote configuration menu, select "Storage>Storage Setting"

,

- Select the recording duration of each video file as desired, with a maximum support of 30 minutes. And you need to select 'Record Type' as' only Video ', and then set the parameters for time-lapse photography in the Time Lapse option. After setting the parameters,

- Enter Storage Devices to check the SD card and format it. As shown in the following figure

10, After completing the above steps, install the camera at the location you want to shoot, wait for 30 minutes, and then you can open the camera on the search tool to enter the web browser interface to play back and download time-lapse photography recorded video files. As shown in the following figure:

Select the camera and right-click to enter the web browser login interface:：

11,  Login username: admin, password: 123456

12,  After logging in to the camera, click on playback to enter the playback interface, as shown in the following figure:

Click the file search button "File List" to search for time-lapse video files that have been recorded by the camera. Click the download button to download and play on the computer, with a file format of * MP4, as shown in the following figure:

Save the video to your computer and play it (due to the high video traffic caused by the time-lapse feature, web browsers and apps cannot play it, so it must be downloaded to your computer to play it).

We will store the video on the internet, and you can click on the following link to view the playback effect:

https://video.aliexpress-media.com/seller/play/5000195398554.mp4